www.PDHcenter.com                                            PDH Course H140                                      www.PDHonline.org

PDH Course H140
Open Channel Hydraulics III 
- Sharp-crested Weirs
Harlan H. Bengtson, Ph.D., P.E.

2009
PDH Online | PDH Center 
 
5272 Meadow Estates Drive

Fairfax, VA 22030-6658

Phone & Fax: 703-988-0088
www.PDHonline.org
www.PDHcenter.com
Open Channel Hydraulics III - Sharp-crested Weirs
Harlan H. Bengtson, Ph.D., P.E.

COURSE CONTENT

1.      Introduction

This course is intended to be taken after the course, H138, “Open Channel Hydraulics I – Uniform Flow.”  It will be assumed that anyone taking this course is familiar with the major classifications used for open channel flow (steady or unsteady state, laminar or turbulent flow, uniform or non-uniform flow, and critical, subcritical or supercritical flow) and with equations and calculations for uniform open channel flow.

A weir, widely used for measurement of open channel flow rate, consists of an obstruction in the path of flow.  Water rises above the obstruction to flow over it, and the height of water above the obstruction can be correlated with the flow rate.  The top of the weir, over which the liquid flows, is called the crest of the weir.  Two commonly used types of weir are the sharp-crested weir and broad-crested weir.  The sharp-crested weir will be covered in this course.   The emphasis will be on calculations used for flow rate over the various types of sharp-crested weirs, but there will also be information on guidelines for installation and use of sharp-crested weirs.

2.
General Information on Sharp-Crested Weirs

A sharp-crested weir is a vertical flat plate with a sharp edge at the top, over which the liquid must flow in order to drop into the pool below the weir.      Figure 1, on the next page, shows a longitudinal section of flow over a sharp-crested weir.


Some of the terminology used in connection with sharp-crested weirs will be presented and discussed briefly here.  As shown in Figure 1, nappe is a term used to refer to the sheet of water flowing over the weir.  Drawdown, as shown in Figure 1, occurs upstream of the weir plate due to the acceleration of the water as it approaches the weir.  Free flow is said to occur with a sharp-crested weir, when

                      
[image: image17.wmf]

Figure 1. Longitudinal Section, Flow Over Sharp-crested Weir

there is free access of air under the nappe.  The Velocity of approach is equal to the discharge, Q, divided by the cross-sectional area of flow at the head measuring station, which should be upstream far enough that it is not affected by the drawdown.  The condition which occurs when downstream water rises above the weir crest elevation is called submerged flow or a submerged weir.  The equations to be discussed for sharp-crested weirs all require free flow conditions.  Accurate measurement of flow rate is not possible under submerged flow conditions.

The three common sharp-crested weir shapes shown in Figure 2 will be considered in some detail in this course.  These shapes are suppressed rectangular, V-notch, and contracted rectangular.

          
[image: image2.png]T
1

f—2— 1
)74

(2) suppressed rectangular (b) V-notch (¢) contracted rectangular






     Figure 2.  Common Sharp-crested Weir Shapes

The reference for the equations, graphs, etc to be used for calculating flow rate over a weir from measured values such as the head over the weir and various weir parameters, will be the 2001 revision, of the 1997 third edition, of the Water Measurement Manual, produced by the U.S. Dept. of the Interior, Bureau of Reclamation.  Water Measurement Manual is available for on-line use or free download at: http://www.usbr.gov/pmts/hydraulics_lab/pubs/wmm/index.htm .  This reference has very extensive coverage of water flow rate measurement.  It is primarily oriented toward open channel flow, but has some discussion of pipe flow (closed conduit pressurized flow) also.

3.
Suppressed Rectangular Weirs

The suppressed rectangular, sharp-crested weir, shown in Figure 2 (a), has the weir length equal to the width of the channel.  The term suppressed is used to mean that the end contractions, as shown in Figure 2 (c), are suppressed (not present).  

The Kindsvater-Carter method (from ref #3 for this course) is recommended in Water Measurement Manual, as a method with flexibility and a wide range of applicability.  This method, for a sharp-crested rectangular weir, will be presented and discussed here.  A more simplified equation, which works well if particular requirements are met, will then be presented and discussed as another alternative.

The Kindsvater-Carter equation for a rectangular, sharp-crested weir (suppressed or contracted) is:

Q = Ce Le He3/2 




(1)
where:

Q  =  discharge, ft3/s

e  =  a subscript denoting "effective"

Ce  =  effective coefficient of discharge, ft1/2/s

Le  =  L  +  kb
He  =  H  +  kh
In these relationships:

kb = a correction factor to obtain effective weir length

L = measured length of weir crest

B = average width of approach channel, ft

H = head measured above the weir crest, ft

kh = a correction factor with a value of 0.003 ft

The factor kh has a constant value equal to 0.003 ft.  The factor kb varies with the ratio of crest length to average width of approach channel (L/B). Values of kb for ratios of L/B from 0 to 1 are available from Figure 3 (as given in Water Measurement Manual). 

          
[image: image3.png]005

T~
+000
byt oo
0

- 0003
0005 h
0 [iki) 04 080 080 100

value of /B




       Figure 3.  kb as a function of L/B (as given in Water Measurement Manual)

The effective coefficient of discharge, Ce, includes effects of relative depth and relative width of the approach channel. Thus, Ce is a function of H/P and L/B.  Values of Ce may be obtained from the family of curves given in Figure 4 (as given in Water Measurement Manual).   Also, values of Ce may be calculated from the equations given after Figure 4.  

As shown in Figure 1, P is the vertical distance to the weir crest from the approach pool invert.  (NOTE:  The term “invert” means the inside, upper surface of the channel.)

               
[image: image4.png]Rectangular Sharp Crested Weir

250 L8-10
200 U8 -08
. =08
c

. Ut =07

3sm
18 -0

rey
B |-

s0m

HP




 

Figure 4.  Ce as a function of H/P & L/B for Rect. Sharp-crested Weir     

The straight lines in figure 4 have the equation form:


[image: image5.png]@
B+c,
o1






Where the equation constants, C1 and C2, in equation (2) are functions of L/B as shown in Table 1, (from Water Measurement Manual) given below.

Table 1.  Values of C1 & C2 for equation (7)  (from Water Measurement Manual)




L/B

   C1 


  C2





0.2

-0.0087

3.152






0.4

 0.0317

3.164






0.5

 0.0612

3.173






0.6

 0.0995

3.178






0.7

 0.1602

3.182






0.8

 0.2376

3.189






0.9

 0.3447

3.205






1.0

 0.4000

3.220



Although the Kindsvater-Carter equation for a rectangular, sharp-crested weir, seems rather complicated as presented on the last three pages, it simplifies a great deal for a suppressed rectangular weir (L = B or  L/B = 1.0).  Equation (2) becomes:





[image: image6.png]C, = 0.4000(F) +3.220




From Figure 3 (L/B= 1): 
  kb =  - 0.003,    so:    Le  =  L  +  kb   =  L  - 0.003

As given above:      kh = 0.003,    so:
   He  =  H  +  kh   =  H  + 0.003
Substituting for Ce,  Le,  & He  into equation (1), gives the following Kindsvater-Carter equation for a suppressed rectangular, sharp-crested weir:


[image: image7.png]3/2

Q= (0.4000(%) +3.220)(L - 0.003)(H + 0.003) 3)





(Keep in mind that equation (3) is a dimensional equation with Q in cfs and               H, P, & L in ft.)

The Bureau of Reclamation, in their Water Measurement Manual, gives equation (4) below, as an equation suitable for use with suppressed rectangular, sharp-crested weirs if the accompanying conditions are met:  

(U.S. units: Q in cfs,  B & H in ft):
Q  =  3.33 B H3/2     

(4)
To be used only if:     

          


[image: image8.png]=033



       
Converting equation (4) to S.I. units gives the following:

(S.I. units: Q in m3/s, B & H in m):
     Q  =  1.84 B H3/2     

(5)
To be used only if:     

          


[image: image9.png]=033



       
Example #1:  There is a suppressed rectangular weir in a 2 ft wide rectangular channel.  The weir crest is 1 ft above the channel invert.  Calculate the flow rate for H = 0.2,  0.3,  0.4,  0.8, 1.0 ft,  & 1.25 ft, using equations (3) & (4).  

Solution:  From the problem statement, L = B = 2 ft, and P = 1 ft, so it is necessary simply to substitute values into the two equations and calculate Q for each value of H.  The calculations were made using an Excel spreadsheet.  The results are shown in the table below, where Q3 is calculated from equation (3) and Q4 is calculated from equation (4).

  H, ft:
0.2
     0.3           0.4
       0.8
   1.0

1.25   (= H/P)

Q3, cfs:       0.603      1.112       1.727         5.087       7.262        10.42
Q4, cfs:       0.596      1.094       1.685         4.766       6.660        9.308

   H/B:
0.1
     0.15
0.2
        0.4
    0.5
         0.625

Discussion of results:  Since P = 1 ft in this example, the values for H in the table of results are equal to H/P.  The values for H/B are shown in the table also.  

	[image: image1.png]drawdown

L
=~ nappe

Zae






	
	Note that the results for equation (3), the Kindsvater-Carter

	
	
	equation, and equation (4) the simpler equation agree quite

	
	
	closely for the three smaller values of H.  They are the

	
	
	same to two significant figures for those three H values.

	
	
	The two conditions for the use of equation (4) are met only

	
	
	for the two smallest H values.  For H = 0.4 ft, H/P is just

	
	
	above the allowable maximum of 0.33.  For the three

	
	
	largest values of H, both H/P and H/B are larger than the

	   Wow, is this all
	allowable values for use of equation (4), and the 
	

	  really discussion
	equation (4) results are quite a bit different from those

	       of results ?
	for equation (5).
	
	
	
	
	


Conclusions:  For a suppressed rectangular, sharp-crested weir, equation (4),      Q  =  3.33BH3/2, may be used if  H/P  <  0.33  &  H/B  <  0.33.  For H/P > 0.33   or H/B > 0.33, the Kindsvater-Carter equation [equation (4)] should be used.

4.
Contracted Rectangular Weirs
The contracted rectangular sharp-crested weir, shown in Figure 2c and in the figure below, has weir length, L, less than the width of the channel, B.  This is sometimes called an unsuppressed rectangular weir.  

                                             
[image: image10.png]L

L—

contracted rectangular weir




For a contracted rectangular, sharp-crested weir (L/B < 1), the Kindsvater-Carter equation becomes:




[image: image11.png]Q=@ (§) + CHT - 0.003(H + k)™ ©)




Where:     C1 and C2  come from Table 1 and  kb comes from Figure 3, for a known value of L/B.  (Table 1 and Figure 3 are on pages 4 & 5.)

The Bureau of Reclamation, in its Water Measurement Manual, gives equation (7) below as a commonly used, simpler Equation for a fully contracted rectangular weir, subject to the accompanying conditions:

(U.S. units: Q in cfs,  L & H in ft):     Q  =  3.33(L – 0.2 H)(H3/2)   
  (7)

The equivalent equation for S.I. units is:

(S.I. units: Q in m3/s, L & H in m):     Q  =  1.84(L – 0.2 H)(H3/2)   
  (8)

For both equation (7) and equation (8), the following conditions must be met:

      i)  Weir is fully contracted, i.e.:     B – L  >  4 Hmax   and    P  >  2 Hmax   

      ii)     H/L < 0.33
Example #2:  Consider a contracted rectangular weir in a rectangular channel with B, L, H, & P having each of the following sets of values. 

a)  Determine whether the conditions for use of equation (7) are met for each set of values.  

b) Calculate the flow rate, Q, using equations (6)  &  (7) for each set of values.

i)      B  =  4 ft,       L  =  2 ft,     H  =  0.5 ft,      P  =  1 ft

ii)     B  =  10 ft,     L  =  6 ft,     H  =  0.8 ft,     P  =  2 ft

iii)    B  =  10 ft,     L  =  4 ft,      H  =  1 ft,       P  =  2.4 ft

iv)    B  =  10 ft,     L  =  4 ft,      H  =  2 ft,       P  =  2 ft

v)     B  =  10 ft,     L  =  8 ft,      H  =  2 ft,       P  =  1.5 ft

Solution:   a)  The required conditions for use of equation (7) are:                           

1)  B – L  >  4 Hmax,       2)  P >  2 Hmax    &      3)  H/L < 0.33

i)  
B – L  =  2 ft  &  4 H  =  2 ft,  so condition 1 is barely met

P = 2 ft  &  2 H = 2 ft,  so condition 2 is barely met

H/L = 0.5/2  =  0.25  <  0.33,  so condition 3 is met
ii)  
B – L  = 4 ft  &  4 H = 3.2 ft,  so condition 1 is met

P = 2 ft  &  2 H = 1.6 ft,  so condition 2 is met

H/L = 0.8/6  =  0.13  <  0.33,  so condition 3 is met
iii)  
B – L  = 6 ft  &  4 H = 4 ft,  so condition 1 is met

P = 2.4 ft  &  2 H = 2 ft,  so condition 2 is met

H/L = 1/4  =  0.25  <  0.33,  so condition 3 is met
iv)  
B – L  = 6 ft  &  4 H = 8 ft,  so condition 1 is not met

P = 2 ft  &  2 H = 4 ft,  so condition 2 is not met

H/L = 2/4  =  0.5  >  0.33,  so condition 3 is not met
v)  
B – L  = 2 ft  &  4 H = 8 ft,  so condition 1 is not met

P = 1.5 ft  &  2 H = 4 ft,  so condition 2 is not met

H/L = 2/8  =  0.25  <  0.33,  so condition 3 is met
	[image: image16.wmf]

	
	
	b)  The calculations for part b) were done with an

	
	
	
	Excel spreadsheet.  The results are summarized in

	
	
	
	the table below.  The flow rates from equation (7)

	
	
	
	was calculated using the given values for L, P & H.

	
	
	
	In order to use equation (6), values were needed

	
	
	
	for C1,  C2,  &  kb.  All three are functions of L/B.

	
	
	
	Values for C1, & C2 were obtained from Table 1,

	
	
	
	and values for kb were obtained from Figure 3.

	        Hey, This is a pretty 
	Q6  &  Q7,  are the flow rates from equations 

	       long solution isn't it ?
	(6) & (7) respectively.
	
	
	





 i)

 ii)

iii)

iv)

v)
eqn (7) ok?
         yes
         yes
         yes

no

no


  L/B


0.5

0.6

0.4

0.4

0.8

  C1

        0.612
       0.0995
       0.0317
       0.0317
      0.2376

  C2

        3.173
       3.178
       3.164
       3.164
      3.189

  kb

        0.010
       0.012
        0.009
       0.009
      0.014

 Q6, cfs
        2.330          14.12
       12.87
       36.37
      80.13

 Q7, cfs
        2.237          13.92
       12.65
       33.91
      71.58

Discussion of Results:  Both equations give similar results for cases i), ii), & iii), where the criteria for use of the simpler Equation (7) were met  (B – L  >  4 Hmax,   P >  2 Hmax,   &    H/L < 0.33).  The results for the two equations differ considerably for cases iv) and v), where the criteria were not met.  

Conclusions:  For a contracted rectangular, sharp-crested weir, the simpler equation [Q  =  3.33(L – 0.2 H)(H3/2) ], appears to be adequate when the three criteria mentioned above are met.  This equation is also acceptable to the U.S. Bureau of Land Reclamation for use when those criteria are met.  The U. S. Bureau of Land Reclamation recommends the use of the slightly more 

complicated Kindsvater-Carter equation [equation (6)]  for use when any of the three criteria given above for use of the simpler Equation (7) are not met.

5.
V-Notch Weirs
The V-notch, sharp-crested weir (also called a triangular weir), shown in Figure  2 (b) and in Figure 5, below, measures low flow rates better than the rectangular weir, because the flow area decreases as H decreases and reasonable heads are developed even at small flowrates.  

The Bureau of Reclamation, in their Water Measurement Manual, gives equation (9) (see below) as an equation suitable for use with a fully contracted, 90o           V-notch, sharp-crested weir if it meets the indicated conditions.   

(U.S. units: Q in cfs,  H in ft)

Q  =  2.49 H2.48     

(9)
Subject to:   P  >  2 Hmax,     S  >  2 Hmax,     0.2 ft  <  H  <  1.25 ft
Where:   
Hmax   =  the maximum head expected over the weir

P        =  the height of the V-notch vertex above the channel invert

S   
=  the distance from the channel wall to the to the V-notch 

    edge at the top of the overflow

See Figure 5, below, for additional clarification of the parameters S,  P,  &  H.

                             
[image: image12.png]





       Figure 5.  Fully Contracted V-notch Weir

A 90o V-notch weir is better than a rectangular weir for measuring relatively low flow rates and a rectangular weir is better for measuring higher flow rates.  This will be illustrated with the next couple of examples.

Example #3:  Calculate the minimum and maximum flow rates covered by the recommended range of 0.2 ft to 1.25 ft for the head over a fully contracted 90o V-notch weir.  (Note:  in order to be fully contracted, P = 2 Hmax   =  2.5 ft.)

Solution:  Substituting values of H into equation (9) gives:

Qmin  =  (2.49)(0.22.48)    =   0.046 cfs  =  Qmin  


Qmax  =  (2.49)(1.252.48)  =   4.33 cfs  =  Qmax  

Example #4:  Calculate the minimum and maximum flow rates covered by the range of 0.2 ft to 1.25 ft for the head over a suppressed rectangular weir in a 2 ft wide rectangular channel.  Assume that P = 2.5 ft, as with the V-notch example, so that the suppressed rectangular weir is fully contracted from the channel bottom.

Solution:  Equation (4) can be used for H = 0.2 ft, because H/P < 0.33 and H/B < 0.33, however for H = 1.25 ft, H/B & H/P are both greater than 0.33, so equation (3) must be used.  The calculations are shown below:

Qmin  =  (3.33)(2)(0.23/2)  =   0.596 cfs  =  Qmin  


Qmax  =  [3.32 + 0.40(1.25/2.5)](2 – 0.003)[(2 + 0.003)3/2]  =   10.42 cfs  =  Qmax  

Comments:  As shown by Example #3 and Example #4, the 2 ft rectangular weir can carry more than twice the flow rate that the V-notch weir can carry for the same head above the channel bottom, however the V-notch weir can measure a much smaller flow rate than is possible with the rectangular weir.

Notch angles other than 90o in a V-notch, sharp-crested weir require the use of the Kindsvater-Carter equation, as given in Water Measurement Manual:


[image: image13.png]Q = 428 Ce Tan() g + K (10)




Where:
Q  =  discharge over weir, cfs



Ce  =  effective discharge coefficient



 H  =  head on weir in ft



 k  =  head correction factor



 =  angle of V-notch

Ce, the effective weir coefficient, and k, the head correction factor, are both functions of only if the V-notch weir is fully contracted (P  >  2 Hmax  &           S  >  2 Hmax).  See Figure 5 on page 13, for clarification of the parameters P and H.  Ce can be obtained from figure 6* or equation (11)*, and k, can be obtained from figure 7* or equation (12)*.  

*from LMNO Engineering, Research and Software, Ltd website, at:  http://www.lmnoeng.com/Weirs 

            
[image: image14.wmf]V-notch Weir Effective Coefficient

0.5750

0.5800

0.5850

0.5900

0.5950

0

20

40

60

80

100

120

Notch angle, 

q 

Ce


  Figure 6.  Effective V-notch Weir Coefficient, Ce

  Ce  =  0.607165052 – (0.000874466963) + (6.10393334 x 10-6)2  

(11)

                 
[image: image15.png]V-notch Weir Head Correction Factor

o0

oo &

k oo

0.004 \

0002

\';0

0 20 40 60 El 100 120
Notch Angle, &







      Figure 7.  V-notch Weir, Head Correction Factor, k

k  =  0.0144902648 – (0.00033955535) 

+ (3.29819003 x 10-6)2 – (1.06215442 x 10-8)

Example #5:  Estimate the flow rate through a fully contracted V-notch weir for a head of 0.2 feet and for a head of 1.25 feet.

a) with a notch angle of 60o 

b) with a notch angle of 40o
Solution:  Part a)  From equation (11) & (12), with   =  60o:  

Ce = 0.5767  &   k = 0.0037

Substituting into equation (10), with H = 0.2 ft, gives:    ( = 60o, H = 0.2 ft)

Q  =  (4.28)(0.5767)[tan(30o)] [(0.2 + 0.0037)5/2]  =  0.0267 cfs

Substituting into equation (10), with H = 1.25 ft, gives:    ( = 60o, H = 1.25 ft)

Q  =  (4.28)(0.5767)[tan(30o)] [(1.25 + 0.0037)5/2]  =  2.508 cfs

Part b)  From equation (11) & (12), with   =  40o:  Ce = 0.5820  &  k = 0.0051

Substituting into equation (10), with H = 0.2 ft, gives:    ( = 40o, H = 0.2 ft)

Q  =  (4.28)(0.5820)[tan(20o)] [(0.2 + 0.0051)5/2]  =  0.0173 cfs

Substituting into equation (10), with H = 1.25 ft, gives:    ( = 40o, H = 1.25 ft)

Q  =  (4.28)(0.5820)[tan(20o)] [(1.25 + 0.0051)5/2]  =  1.600 cfs

Comment:   This example is intended to simply illustrate the use of equations (10), (11), & (12) for   V-notch weirs with notch angles other than 90o.   

6.
Installation & Use Guidelines for Sharp-Crested Weirs
A summary of installation and measurement guidelines for sharp-crested weirs extracted from Water Measurement Manual, is given in this section.

a)  The upstream face of the weir plates and bulkhead should be plumb, smooth, and normal to the axis of the channel.

b)  The entire crest should be level for rectangular and trapezoidal shaped weir openings, and the bisector of V-notch angles should be plumb.

c)  The top thickness of the crest and side plates should be between 0.03 and 0.08 inch.

d)  The upstream edges of the weir opening plates must be straight and sharp. Edges of plates require machining or filing perpendicular to the upstream face to remove burrs or scratches and should not be smoothed off with abrasive cloth or paper. Avoid knife edges because they are a safety hazard and damage easily

e)  The overflow sheet or nappe should touch only the upstream faces of the crest and side plates.

f)  Maximum downstream water surface level should be at least 0.2 ft below crest elevation. However, when measuring close to the crest, frequent observations are necessary to verify that the nappe is continually ventilated without waves periodically filling the under nappe cavity.

g) The measurement of head on the weir is the difference in elevation between the crest and the water surface at a point located upstream from the weir a distance of at least four times the maximum head on the crest.

h)  The approach to the weir crest must be kept free of sediment deposits.

i)  If the weir crest length is greater than 50% of the approach channel width, then ten average approach flow widths of straight, unobstructed approach are required.

j)  If the weir crest length is less than 50% of the approach channel width, then twenty average approach flow widths of straight, unobstructed approach are required.

k)  If upstream flow is below critical depth, a jump should be forced to occur.  In this case, thirty measuring heads of straight, unobstructed approach after the jump should be provided.

l)  The minimum head over the weir should be 0.2 ft.

For Rectangular, Sharp-crested Weirs:

All of the general conditions, a) through l) above, apply.  Also:

a)  The crest length, L, should be at least 6 inches.

b)  The crest height, P, should be at least 4 inches.

c)  Values of H/P should be less than 2.4.

For V-notch Weirs:

All of the general conditions, a) through l) above, apply.  Also:

a)  For a fully contracted V-notch weir, the maximum measuring head should be less than 1.25 ft.

b)  For a fully contracted V-notch weir, H/B should be less than 0.2.

c)  The average width of the approach channel, B, should always be greater than 3 ft for a fully contracted V-notch.

d)  The V-notch of the weir should always be at least 1.5 ft above the invert of the weir pool for a fully contracted V-notch.

7.
Summary

Sharp-crested weirs are commonly used for flow rate measurement in open channels.  Three types of sharp-crested weirs:  suppressed rectangular, contracted rectangular, and V-notch, are covered in detail in this course.  Emphasis is on calculation of flow rate over a weir for given head over the weir and weir/channel dimensions.  For each of the three types of sharp-crested weir, a general equation with a wide range of applicability is presented and discussed along with equations and/or graphs as needed for use with the main equation.  Also, for each of the three types of sharp-crested weir, a simpler equation is presented along with a set of conditions under which the simpler equation can be used.  Several worked examples are included covering all three types of weirs.  Practical installation and use guidelines for sharp-crested weirs are presented.





































An Approved Continuing Education Provider
PAGE  
©2009 Harlan H. Bengtson                                                                                                         Page 2 of 20

_1295503588

_1295504356

_1296286692

_1296288704

_1296288949.xls
Chart1

		20

		40

		60

		80

		100



Notch angle, q

Ce

V-notch Weir Effective Coefficient

0.5921172861

0.5819526668

0.5766711942

0.5762728683

0.5807576891



Sheet1

		

										Cwt

						90o		60o		45o		20o

				0.2		0.602		0.612		0.621

				0.25		0.597		0.607		0.614		0.647

				0.3		0.593		0.602		0.609		0.638

				0.35		0.59		0.599		0.606		0.631

				0.4		0.588		0.596		0.602		0.626

				0.5		0.585		0.591		0.597		0.619

				0.6		0.583		0.588		0.593		0.613

				0.7		0.582		0.586		0.59		0.609

				0.8		0.581		0.584		0.588		0.606

				0.9		0.581		0.583		0.586		0.603

		C = 0.607165052 - 0.000874466963 Ø  +  6.10393334x10-6 Ø2

		k (ft.) = 0.0144902648 - 0.00033955535 Ø  + 3.29819003x10-6 Ø2  - 1.06215442x10-8 Ø3

		where Ø is the notch angle in degrees

				q		Ce				q		k

				20		0.5921				20		0.0089334615

				40		0.5820				40		0.005505376

				60		0.5767				60		0.0036961744

				80		0.5763				80		0.0029960224

				100		0.5808				100		0.0028950859





Sheet1

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0

		0		0		0		0



H, ft

Cwv

V-notch Weir Coefficient

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0



Sheet2

		0

		0

		0

		0

		0



Notch angle, q

Ce

V-notch Weir Effective Coefficient

0

0

0

0

0



Sheet3

		0

		0

		0

		0

		0



Notch Angle, q

k

V-notch Weir Head Correction Factor

0

0

0

0

0



		





		





Q-423¢C lan( j (n+P’?

where Q = Discharge (cf9)
C=Discharge Coefficient
Notch Angle
h=Head (f)

cad Correction
Factor (fr)

Candk vs. Notch Angle

06 001
05 0008
056 =C 0006 &

“hsa o004
052 k= 0.002
0s o

g W & B 100

Notch Angle (degrees)







_1295604694

_1295503966

_1295502285

_1295502803

