Practical Multi-Factor Test and Analysis

Quiz Questions

- Multi-Factor testing can identify interactions between controlled factors.

  - True

  - False

- Equipment specifications are not an important consideration in test design.

  - True

  - False

- Uncontrolled factors like weather conditions can be ignored.

  - True

  - False

- The variance of a population is

N

σ2 = ∑(Sn-μ)2/(N-1)

n=1

  - True

  - False

- The Sample Standard Deviation is the square root of the Sample Variance.

  - True

  - False

- In a normally distributed population, what percentage will be within 1 standard deviation of the mean?

  - 95.4 %

  - 50.2 %

  - 45.7 %

  - 68.2 %

- ANOVA is an acronym for

  - Algebraic Number Variation Analysis

  - Alternative Numeric Variance Analysis

  - Analysis of Variance

  - Action Notation of Variation

- The null hypothesis of an ANOVA is

  - All the treatment will give different results.

  - All the treatments will yield the same results.

  - The F ratio will equal 1.

  - The F ratio will be less than 1.

- A factor tested at two levels has how many degrees of freedom?

  - 2

  - 0

  - 1

  - 3

- What is the error term degrees of freedom for a 23 test with 3 replications?

  - 16

  - 24

  - 8

  - 4

- A 23 test with 3 replications indicates a contrast of 16.7 for factor A. What is the Sum of Squares for factor A?

  - 16.7

  - 2.0875

  - 11.62

  - Not enough information.

- The mean squares for factor A above is

  - 16.7

  - 2.0875

  - 11.62

  - Not enough information.

- A calculated probability for a factor or interaction of 0.05 indicates

  - A 5% probability of the factor changing the test response.

  - A 5% probability of the result occurring randomly.

  - A 5% chance of success.

  - A 5 % chance of interaction.

- The Total Sum of Squares for a 23 test with 2 replications and a standard deviation on all data of 15.8 is

  - 3744.6

  - 237

  - 16.64

  - 1.05

- A normal probability plot of residuals resulting in a straight line indicates a normal distribution and aids in validating the test data.

  - True

  - False

A test of vibration levels in a milling process is proposed to test bit size, RPM and feed rate with the objective to minimize the vibration level.  The factors are as follows:

|  | A: Bit Diameter | B : RPM | C: Feed Rate in/min |
| --- | --- | --- | --- |
| - | 1/8 | 250 | 2 |
| + | ¼ | 400 | 3.5 |

3 replications are run for each test with the data as follows:

| Treatment | REP 1 | REP 2 | REP 3 |
| --- | --- | --- | --- |
| 1 | 25.025 | 17.7375 | 22.75 |
| 2 | 37.4 | 30.8 | 34 |
| 3 | 21.8625 | 20.7625 | 19.875 |
| 4 | 56.375 | 49.9125 | 51.25 |
| 5 | 25.9875 | 19.8 | 23.625 |
| 6 | 33 | 30.9375 | 30 |
| 7 | 19.9375 | 19.525 | 18.125 |
| 8 | 60.3625 | 54.8625 | 54.875 |

Use the 2 x3 template to fill in the data and perform the calculations for the answers to questions 16-xx.

- What factors and interactions are calculated to be Statistically Significant at the 95% confidence level?

  - Bit Diameter and Feed Rate

  - Bit Diameter and RPM

  - Bit Diameter, RPM, Bit Diameter and RPM Interaction, Bit Diameter and RPM and Feed Rate Interaction.

  - Bit Diameter, Feed Rate, RPM and Feed Rate Interaction

- The largest effect calculated is for

  - Bit Diameter

  - RPM

  - Feed Rate

  - Bit Diameter and RPM Interaction

- Treatment 4 conditions are

  - 1/8 Bit Diameter, 250 RPM and 2 in/min feed rate

  - 1/4 Bit Diameter, 250 RPM and 3.5 in/min feed rate

  - 1/8 Bit Diameter, 400 RPM and 2 in/min feed rate

  - 1/4 Bit Diameter, 400 RPM and 2 in/min feed rate

- The error Mean Squares is

  - 120.98

  - 7.56

  - 4634.3

  - 16

- The lowest resulting vibration would be expected to be

  - 1/8 Bit Diameter, 250 RPM and 2 in/min feed rate

  - 1/4 Bit Diameter, 250 RPM and 3.5 in/min feed rate

  - 1/8 Bit Diameter, 250 RPM and 3.5 in/min feed rate

  - 1/4 Bit Diameter, 400 RPM and 3.5 in/min feed rate

- The Signal to Noise Ratio for the Bit Diameter / RPM interaction is

  - 892

  - 38.5

  - 118

  - 12.2

- The cumulative probability point (Pk) for the residual of treatment 5, rep 1 is

  - .604

  - .062

  - .479

  - .813

- The rank of residual for treatment 4, rep 2 is

  - 22

  - 4

  - 2

  - 1

- The residuals plot for this test is a roughly linear line and indicates a normal distribution

  - True

  - False

- The mean value for vibration with a ¼ diameter bit, 400 RPM and 3.5 in/min feed rate is

  - 33.7

  - 56.7

  - 21.8

  - 31.2